Date – 17-Jan-24

Issue – TE-10300: BMS VC5: frame 594 not emitted after key on.

Issue Description:

Just after a key on, BMS stop emission of 594 CAN frame during around 20sec. Then this frame is emitted again during 2.6 sec (seems to be because of HV_BATT_WATER_TEMP variation, even if not normal because this parameter is not an trigger event related to this mixt frame…), then stopped again during 43sec. After that, 594 frame take back its normal periodic emission of 1sec.All other TBMU frames are well sent during this period.

Power Supply

BSW SW

OS Task Priority

EB Tresos Configuration for 0x594

Sudden Voltage drop

TE-10300 : BMS VC5 : frame 594 not emitted after key on

High Priority Frames

Temp below -5dC

Error Frames

High Bus Load

Ambient Temperature

CAN Network

| Cause |  | Analysis performed | Result / Remark |
| --- | --- | --- | --- |
| CAN Network | High Priority Frames | Checked high priority frames in given issue log. High priority frames are observed every 1sec so it will not cause any issue to low priority message | Pass |
|  | Error Frames | Transmitted error frames | Pass |
|  | High Bus Load | As per CAN log file provided by customer there in no CAN BUS load observed. | Pass |
| BSW SW | EB Tresos Configuration for 0x594 | Cross verify the configuration of 0x594 message with similar mixed frame 0x794 and only the changes we found ComTxMode Time offset. Need to do further analysis. | In progress. Completing by 18th Jan |
|  | OS Task Priority | Invoking ComTxMainfunction in interrupt function, still issue is not reproducible. Need to check with different task which has lower timing value. | In progress. Completing by 18th Jan |
| Power Supply | Sudden Voltage drop | Change the ECU power supply voltage from 12v to 5v and back to 12v manually. Repeat this multiple times. | - There is no frame loss after Voltage spike. - If voltage drops below 5V, ECU gets reset and transmission of all messages stop. - Once supply voltage returns to 12v, CAN message transmission resumes. Pass |
| Ambient Temperature | Temp below -5dC |  |  |